Governing Equations of Fluid Flow

| Governing Equation | Mathematical Form | What Physical Law It Represents | What It Governs / Provides |
| --- | --- | --- | --- |
| Continuity Equation |  | Conservation of mass | Determines how density and velocity fields must relate so mass is conserved |
| Incompressible Continuity |  | Volume conservation for constant density | Constrains the velocity field so fluid elements cannot change volume |
| Momentum Equation (Navier–Stokes) |  | Newton’s Second Law applied to a fluid | Determines the velocity field and pressure distribution |
| Energy Equation |  | Conservation of energy | Governs temperature distribution, heat transfer, and thermal energy |
| Equation of State |  | Thermodynamic relation between properties | Relates pressure, density, temperature |
| Species Transport Equation |  | Conservation of chemical species | Determines concentration fields of dissolved species |
| Vorticity Equation (derived from NS) |  | Rotation dynamics of the flow | Governs vorticity and rotational motion |
| Boussinesq Momentum Equation | _____________________________________________________ | Momentum with buoyancy due to temperature variations | Determines velocity and pressure in buoyancy-driven flows |

What these equations collectively solve for

In a typical fluid flow problem, these equations determine the following fields:

| Flow Property | Provided By |
| --- | --- |
| Velocity field | Navier–Stokes + continuity |
| Pressure field | Navier–Stokes |
| Density | Continuity + equation of state |
| Temperature | Energy equation |
| Species concentration | Species transport equation |
| Vorticity ____________________________________________ | Derived from velocity |

The "core four" used most often

For many problems (especially incompressible flows), the governing system reduces to:

- Continuity

- Navier–Stokes (3 momentum equations)

- Energy equation (if heat transfer matters)

- Equation of state (if compressible)

These equations together form a closed system for the unknown flow fields.

💡 Important conceptual point

Fluid mechanics is essentially solving for fields:

[
\mathbf{u},; p,; \rho,; T,; C
]

and each governing equation enforces a fundamental conservation law.

If you'd like, I can also show a second table that fluid mechanics professors love that maps each equation directly to its control-volume conservation law (mass, momentum, energy) and the terms that appear in them. That one makes the whole structure of fluid mechanics click instantly.